LED Ping-Pong

Adam Turičík

Richard Vaško

Václav Vojtovič

Authors

https://github.com/vojtovic/LED-Ping-Pong-DE1-project

Github link:

This project implements a Ping-Pong game on the Nexys A7-50T development board. A single LED, which is lit, represents the ball and moves across a 16-LED array. When the ball reaches the edge, the player must press the corresponding button (left button for the left edge, right button for the right edge).  The ball speed increases after each successful hit. The score is displayed on 7-segment display as decimal number. To restart after game over, the reset button must be pressed.

Project Summary

The game start in the centre of the LED array (num. 7). When the ball arrives to the very last position on each edge, a player must press the corresponding button in due time to “hit” the ball.  After each successful hit, accompanied by a flash of the green LED, the ball changes direction and speeds up. If a player doesn´t make it in time, the red LED is lit, signalizing the end of the game. The game can then be reset by pressing the centre button.

Rules

| Signal name | I/O | Size | Note |
| --- | --- | --- | --- |
| clk | in | 1 | system clock (10 MHz) |
| btnc | in | 1 | system reset (centre button) |
| btnr | in | 1 | right button |
| btnl | in | 1 | left button |
| led | out | 15:0 | LED array (ball position) |
| led16_g | out | 1 | green LED (successful hit) |
| led16_r | out | 1 | red LED (miss / game over) |
| seg | out | 6:0 | 7-segment display segments |
| an | out | 7:0 | 7-segment display anodes |

Inputs and outputs

- Debounce

- Front counter

- Back counter

- Clock enable

- Bin2led

- Score counter

- Led

- Display driver

Components

Game starts when a player presses one of the buttons. The Debounce component eliminates any bounce effects, which are created when a button is pressed.  Multiplexer switches between counters, determining the direction of the ball. Speed of the ball is determined by the Speed counter component which speeds up the output signal of clk_en. Clock enable slows down the clock signal for human reactions. Component Bin2led converts binary numbers to a 16-bit one-hot code, which lights up exactly one LED corresponding to the ball´s position. Score is counted by the Score counter module and is displayed through the Display driver component on the 7-segment display.

Description

Architecture of this project is using 445 look-up tables, 129 flip flops, 37 input-output (this is number of used pins on FPGA chip). Total chip consumption for this architecture is 98 mW.

Space on FPGA

Image link: https://www.magnific.com/free-photos-vectors/ping-pong-ball-illustration

Sources

Top level schematic

Graph of power comsuption